Histone Remote Management System Windows Application User Guide

Qingdao Histone Intelligent Commercial System Co.,Ltd

Content

1.	Software installation	3

2.	Matters need attention	3

3.	Register	4

4.	Application function introduction	6

4.1.	Hardware Manager	6

4.2.	Device Information	7

4.3.	Troubleshooting	7

4.3.1.	One-click Test	8

4.3.2.	Comprehensive Test	8

4.3.3.	Device Problem	9

4.3.4.	Remote Assistance	10

4.3.5.	File Upload	10

4.4.	Setup Settings	11

4.5.	Notification	12

4.6.	Guidelines	12

5.	Advertising Usage Introduction	13

5.1.	Configure advertising	13

5.2.	Advertising version introduction	14

5.3.	Advertising call method	14

5.4 Advertising demo usage instructions	14

- Software installation

This software does not need to be installed. Double-click the HiAssistant.exe to run it, the exe file is in the HiAssistant_release folder. The operating environment requires win7 or above, compatible with 64-bit and 32-bit systems.

To avoid affecting the use of other software, the software will automatically hidden in the taskbar after running(lower right corner of OS desktop). To use the software, click the "Hi" icon in the lower right corner of the screen to pop up the User interface.

Note: After the software is running, two icons  will appear in the lower right corner of the system, where the software is running in the background, there is no visual interface, and you should not close it without permission.

Click the icon to pop up the user interface. When customer want to exits, right-click the icon and select the exit button to automatically exit the program, then  will also quit.

- Matters need attention

- Program storage directory cannot contain Chinese;

- Control panel \ System and Security \ Security and Maintenance \ Change User Account Control Settings to "never notify";

- Before the software is used, it is necessary to manually add the software to whitelist. The software to be added includes hiassistant.exe, himonitor.exe (the process under Win10 is called HiAssistant);

- When using the software, the driver protection function of 360 security guard or other similiar software should be turned off;

- If the firewall warning message of 360 security software(or other similar security software) pops up, please select the program as trusted and allow the program to run;

- Need set the “Use Unicode UTF-8 for worldwide language support”, then restart the device.set steps as below picture shows:

- Register

Register the device when the first time you run HiAssistant.exe, click the above icon on lower right corner of OS desktop.

First fill in the organization code, select the device type, and then click the submit button.

Fill in the device name, click the "register" button, and the registration is complete. If you need to modify the organization code, you can click the "previous step" button to modify.

- Application function introduction

The software home page displays basic data and function buttons. The basic information includes the current running time, the entire machine usage time, CPU utilization, memory and hard disk usage, equipment and network status, etc .; the function buttons include hardware manager, equipment information, troubleshooting, setup settings, message notification and operation guide.

Hardware Manager

Click the "hardware manager" button on the home page, and the home page of hardware manager will pop up. Hardware manager includes system information, cash drawer, printer, monitor, touch screen, speaker, RFID card, MSR card and other functions.

Device Information

The device information interface displays POS overview, processor, storage, mainboard, operating system, process, install software and other information.

Troubleshooting

Troubleshooting includes one-click test, comprehensive test, Device problems, remote assistance, and file upload functions.

    - One-click Test

Click the "One-click test" button on the troubleshooting homepage to start one-click test. Click the "Save Results" button, and the test results will be saved in the testResult.txt file in the root directory of the local program. Click the "Results Upload" button, and the test results will be uploaded to the server.

    - Comprehensive Test

Click the "Comprehensive Test" button on the troubleshooting homepage to start a comprehensive test. Click the "Save Results" button to save the comprehensive test results in the testResult.txt file in the root directory of the local program. Click the "Results Upload" button to upload the comprehensive test results to the server.

    - Device Problem

Click the "Device Problems" button on the troubleshooting homepage to enter the device problem interface, select the fault classification, fill in the fault description, click the fault level, and click the "Fault Report" button to upload the local fault to the server.

Click the "common problem" button to display a list of frequently asked questions.

    - Remote Assistance

Click the "Run remote control" button to request remote assistance from the server; click the "Cancel Remote control" button to cancel requesting remote assistance to the server; click the "Current Request NO." button to display the current number of requests for server remote assistance.

    - File Upload

Click the "Browse…" button to select the file to be uploaded, and fill in the description in the file description blank. Click the "Upload Files" button to upload the file. Click the "Cancel Upload" button to cancel uploading the file.

Setup Settings

The Setup settings interface is used to display and modify configuration information, including the basic settings and peripheral settings. The basic settings are divided into server information and organization information; peripheral settings include peripheral configurations such as printers and cash drawers. When modifying the configuration, fill in the changes and click the "Change" button.

Note:

1. The gray blank content is for display only and cannot be modified;

2. After clicking the "Change" button, the configuration needs to restart the program to take effect.

Notification

The message notification interface is used to display messages delivered by the server.

Guidelines

The guidelines interface is used to display the user manual.

- Advertising Usage Introduction

- Configure advertising

Open the Config.ini configuration file in the root directory of the program.

Specific as follows:

; HiAssistant configuration

[Config]

; Tray hidden 1 Hidden 0 display

IsTrayHide=0

; Hardware Butler Whether to hide 0 Show 1 hide

IsHsCheck=1

; AD configuration

[HiAd]

; Offline advertising 1 Offline 0 Online

IsLocal=0

; Whether the AD automatically starts 1 self start 0 cancel self start

IsAutoStart=1

; Primary and secondary screens Select 1 Primary and secondary screens

Screen=1

; Whether to display the lower left picture 0 not display 1 Display

showPicture=0

; Enter rgb starting with # for the color displayed

ScreenColor=#d6753b

[HiAd] indicates the AD configuration option.

If you want to set the adsoftware to start upon startup, set IsAutoStart=1, save the Settings, and restart HiAdPlayer.exe to complete the setting.

If you want to set the offline adsoftware, set IsLocal=1, save the modification, and restart HiAdPlayer.exe to complete the offline adsoftware setting.

If you need to set the adware display Screen, set Screen (for exe), save the Settings, and restart HiAdPlayer.exe.

If you need to set the lower left corner of the adware picture display or not, set showPicture (applicable to dll version), save after modification, restart the software can be.

- Advertising version introduction

1. Offline advertising: The offline version only needs to broadcast advertising resources in the ad folder in the root directory of the program without networking environment.

2. Online version of advertising: If a network environment is required, advertising resources and playing rules can be configured and distributed on the advertising front-end web page. After receiving the advertisement, the client will play advertising resources according to the playing rules

- Advertising call method

1. Direct operation: Double-click HiAdPlayer.exe in the root directory of the program to start the adsoftware.

2. Dynamic library call mode: for the convenience of customers, this software provides AD widnows version of dynamic library HiAdPlayer.dll (the dynamic library is located in the root directory of the program), users can directly adjust the dynamic library to use.

Application scenario: Users need to display a list of products while playing advertisements.

Usage: When using the.dll AD client, you need to load the HiAdPlayer.dll dynamic library in the root directory of the program and call the corresponding interface. The interface of the dynamic library is as follows：

1) int HsDisplay_Run(int nScreen)

Function: Run AD client

Parameters: nScreen: 1: home screen; 2: expansion screen

2) void HsDisplay_Plu(const char* PluList)

Function: Send product details to the client

Parameters: PluList: commodity details, parameter naming rules "name of commodity" | | | quantity unit price, such as "| | 1 | rice bowl 2.00".

3) void HsClear_Plu ()

Run the following command to clear the client product list

4) void HsDisplay_Close ()

Function: Close the advertiser

- 5.4 Advertising demo usage instructions

Open AdPlayerDemo.exe in the root directory of the program, and click OK after the dynamic library is loaded successfully. The following figure is displayed.

First select display on the main screen or the secondary screen, and then click the product, which will be displayed on the corresponding screen. The effect picture is as follows：

The displayed AD can be configured in the background service. Click the Clear button and it will be automatically displayed in full screen after 5 seconds.

Note:

1. The picture displayed in the lower left corner should be placed in the adPicture folder under the execution path and named test.png.

2, in the advertising dll version, divided into two display methods: (1) display the bottom left corner of the picture, advertising resources should be suitable for the screen resolution, when the pop-up product list interface, will be reduced to the original 3/4, does not affect the advertising display; ② When the bottom left picture is not displayed, when the product list interface pops up, the left 3/4 of the screen will display advertising resources, and the right 1/4 of the screen will display the product list.

3. In the client version, select resources with the same screen resolution to achieve the full screen effect.